NEW ZEALAND MAP GRID

10xxxxxx.1

# LEGAL RESTRICTIONS:

This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# DESCRIPTION:

NEW ZEALAND MAP GRID is a C language code component that provides conversions between Geodetic coordinates (latitude and longitude) and New Zealand Map Grid projection coordinates (easting and northing).

# CERTIFICATION LEVEL:

This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

# LEVEL OF TESTING/ACCEPTANCE:

Unit and integration testing have been performed for the functions contained in this component.

# PURPOSE/INTENDED USE:

The purpose of NEW ZEALAND MAP GRID is to provide a reusable component that supports the following coordinate conversions:

- Geodetic coordinates (latitude and longitude, in radians) to New Zealand Map Grid projection coordinates (easting and northing, in meters), and

- New Zealand Map Grid projection coordinates (easting and northing, in meters) to Geodetic coordinates (latitude and longitude, in radians).

A particular ellipsoid is specified in terms of the following parameters:

- Ellipsoid Code		Standard 2-letter ellipsoid code (Ellipsoid must be International).

The following ellipsoid and projection parameters are held constant:

- Semi-Major Axis (a):	Radius (in meters) at the equator (6378388.0),

- Central Meridian – Longitude (in radians) at the horizontal center of the projection (173.0 degrees),

- Origin Latitude – Latitude (in radians) at which the scale factor of the projection is 1.0 (-43.0 degrees),

- False Easting – A coordinate value (in meters) assigned to the central meridian of the projection to avoid the inconvenience of using negative coordinates (2510000.0), and

- False Northing – A coordinate value (in meters) assigned to the origin latitude of the projection to avoid the inconvenience of using negative coordinates (6023150.0).

# HARDWARE/ENVIRONMENT CONSTRAINTS:

There are no hardware or environment constraints.  There are no limitations.

# FUNCTIONS:

Set_NZMG_Parameters – This function sets the ellipsoid parameters to be used in subsequent coordinate conversion operations.

Get_NZMG_Parameters – This function returns the current values of the ellipsoid parameters.

Convert_Geodetic_To_NZMG – This function converts the specified geodetic coordinates (latitude and longitude, in radians) to New Zealand Map Grid projection coordinates (easting and northing, in meters).

Convert_NZMG_To_Geodetic – This function converts the specified New Zealand Map Grid projection coordinates (easting and northing, in meters) to geodetic coordinates (latitude and longitude, in radians).

# EXAMPLE ApplicationS:

The following example illustrates how New Zealand Map Grid can be used to convert Geodetic coordinates to New Zealand Map Grid projection coordinates and back again:

## Function Call:

status = Set_NZMG_Parameters (Ellipsoid_Code)

## Inputs:

- Ellipsoid_Code	“IN” (International)

## Outputs:

- None

## Function Call:

status = Convert_Geodetic_To_NZMG (Latitude, Longitude, Easting, Northing)

## Inputs:

- Latitude:	-35.0

- Longitude:	175.0

## Outputs:

- Easting:	2693473.80

- Northing:	6687001.06

## Function Call:

status = Convert_NZMG_To_Geodetic (Easting, Northing, Latitude, Longitude)

## Inputs:

- Easting:	2693473.80

- Northing:	6687001.06

## Outputs:

- Latitude:	-35.0

- Longitude:	175.0